## Mantis 3398

## SV-DC User-defined nets and resolution functions

Version 14 (fixing Shalom’s comments)

## At the end of 6.6 Net types, after section 6.6.6 Supply nets ADD a new section:

### 6.6.7 User-defined net types

A user-defined nettype allows users to describe more general abstract values for a wire, including its resolution function. This nettype is similar to a typedef in some ways, but shall only be used in declaring a net.  It provides a name for a particular data type and optionally an associated resolution function.

Add a new syntax box for net type declaration and renumber if necessary

The syntax for net type declarations is given in Syntax 6-x

net_type_declaration ::=  						// from A.2.1.3

nettype data_type  net_type_identifier

[with  [package_scope|class_scope] tf_identifier];

| nettype [package_scope|class_scope] net_type_identifier net_type_identifier;

Syntax 6-2—Syntax for net declarations (excerpt from Annex A)

A net declared with a nettype therefore uses that data type and, if specified, the associated resolution function. A user-defined nettype has no default type; a specified data type is required. The data type shall include only integral types and real types as well as unpacked structs and fixed size arrays of a legal data type.

A second form of a nettype declaration is to create another name for an existing nettype.

An atomic net is a net whose value is updated and resolved as a whole. A net declared with a user-defined nettype  is an atomic net.  Similarly, a logic net is also an atomic net but a logic vector net is not an atomic net. While an atomic net may have a singular or aggregate value, each atomic net is intended to describe a single connection point in the design.

The resolution for a user-defined nettype is specified using a SystemVerilog function declaration.  If a resolution function is specified, then when a driver of the net changes value, an update event is scheduled on the net in the Active (or Reactive) region. When the update event matures, the simulator calls the resolution function to compute the value of the net from the values of the drivers.  The return type of the function shall match the data type of the nettype.  The function shall accept an arbitrary number of drivers, since different instances of the net could be connected to different numbers of drivers.  Any change in the value of one or more of the drivers shall trigger the evaluation of the resolution function associated with that nettype.

A user-defined resolution function for an atomic net with a data type T shall be a function with a return type of T and a single input argument whose type is a dynamic array of elements of type T. A resolution function shall be automatic (or preserve no state information) and have no side effects.  A resolution function shall not resize the dynamic array input argument nor shall it write to any part of the dynamic array input argument. While a class function method may be used for a resolution function, such functions shall be class static methods as the method call occurs in a context where no class object is involved in the call.  Parameterized variants of such methods can be created through the use of parameterized class methods as described in 13.8.   [Note to editor: 13.8 is a new section in Mantis 696 and should be integrated first; any change to that section number should be reflected here.]

Two different nettypes can use the same data type, but have different resolution functions  A nettype may be declared without a resolution function, in which case it shall be an error for a net of that nettype to have multiple drivers.

Due to non-determinism within scheduling regions, if there are multiple driver updates within a scheduling region, there may be multiple evaluations of the resolution function.

A force statement can override the value of a net of a user-defined nettype. When released, the net returns to the resolved value.

// user-defined data type T

typedef struct {

real field1;

bit field2;

} T;

// user-defined resolution function Tsum

function automatic T Tsum(input T driver[]);

Tsum.field1 = 0.0;

foreach (driver[i])

Tsum.field1 += driver[i].field1;

endfunction

nettype T wT; // an unresolved nettype wT whose data type is T

// a nettype wTsum whose data_type is T and

// resolution function is Tsum

nettype T wTsum with Tsum;

// user-defined data type TR

typedef real TR[5];

// an unresolved nettype ‘wTR’ whose data_type

// is an array of real

nettype TR wTR;

// declare another name nettypeid2 for nettype wTsum

nettype wTsum nettypeid2;

The following example shows how to use a combination of a parameterized class definition with class static methods to parameterize the data type of a user-defined nettype.

class Base #(parameter p = 1);

typedef struct {

real r;

bit[p-1:0] data;

} T;

static function T Tsum(input T driver[]);

Tsum.r = 0.0;

Tsum.data = 0;

foreach (driver[i])

Tsum.data += driver[i].data;

Tsum.r = $itor(Tsum.data);

endfunction

endclass

typedef Base#(32) MyBaseT;

nettype MyBaseT::T narrowTsum with MyBaseT::Tsum;

typedef Base#(64) MyBaseType;

nettype MyBaseType::T wideTsum with MyBaseType::Tsum;

narrowTsum net1;    // data is 32 bits wide

wideTsum net2;      // data is 64 bits wide

### In 6.7, 10.3 and A.2.1.3, CHANGE FROM:

net_declaration ::=

net_type  [drive_strength | charge_strength] [vectored | scalared]

data_type_or_implicit [delay3] list_of_net_decl_assignments;

TO:

net_declaration ::=

net_type  [drive_strength | charge_strength] [vectored | scalared]

data_type_or_implicit [delay3] list_of_net_decl_assignments;

|  net_type_identifier [ #delay_value  |  # ( mintypmax_expression)]

list_of_net_decl_assignments;

## At the end of 6.7 Net declaration, ADD 2 new sections

### 6.7.1 Net Declaration with user-defined nettype

A net with a user-defined nettype allows users to describe more general abstract values for a wire.  A net declared with a nettype uses the data type and any associated resolution function for that nettype.

// an unresolved nettype wT whose data type is T

// The nettype and resolution function declarations are in

// section 6.6.7

nettype T wT;

// a nettype wTsum whose data type is T and

// resolution function is Tsum

nettype T wTsum with Tsum;

// a net of unresolved nettype wT

wT w1;

// an array of nets, each net element is of unresolved nettype wT

wT w2[8];

// a net of resolved nettype wTsum and resolution function Tsum

wTsum w3;

// an array of nets, each net is of resolved nettype wTsum

wTsum w4[8];

// user-defined data type TR which is an array of reals

typedef real TR[5];

// an unresolved nettype ‘wTR’ with data_type TR

nettype TR wTR;

// a net with unresolved nettype wTR and data_type TR

wTR w5;

// an array of nets, each net has an unresolved nettype wTR

// and data type TR

wTR w6[8];

Certain restrictions apply to the data type of a net with a user-defined nettype. A valid data type shall be one of the following:

a) A 4-state integral type, including a packed array or packed structure.

b) A real or shortreal type

c) A fixed-size unpacked array or unpacked structure, where each element has a valid data type for a

net of a user-defined nettype.

### 6.7.2 Initialization of user-defined atomic nets

The resolution function for any resolved atomic net shall be activated at time zero at least once.  This activation occurs even for atomic nets with no drivers or value changes on drivers at time zero.  Since the actual evaluation of the resolution function is subject to scheduling non-determinism, no assumptions can be made regarding the state of driven values during the guaranteed call, which may precede or follow any driver changes at time zero.

The initial value of an atomic net shall be set before any initial or always procedures are started and before the activation of the guaranteed time zero resolution call.  The default initialization value for a user-defined net shall be the default value defined by the data type.   For an atomic net whose data type is a struct type, any initialization expressions for the members within the struct shall be applied.

At the end of 7.2.2 Assigning to structures, modify:

The initial assignment expression within a data type shall be ignored when using a data type to declare a net that does not have a user-defined nettype. (see 6.7).

### In 6.22 Type compatibility, add new section:

### 6.22.6 Matching nettypes

- An atomic nettype matches itself and the nettype of atomic nets declared using that nettype within the scope of the nettype type identifier.

- A simple nettype that renames a user-defined atomic nettype matches that user-defined atomic nettype within the scope of the nettype identifier.

// declare another name nettypeid2 for nettype nettypeid1

nettype nettypeid1 nettypeid2;

### At the end of 7.2.2 Assigning to structures, CHANGE:

The initial assignment expression within a data type shall be used for atomic nets (see 6.6.7 and 6.7.1) but shall be ignored when using a data type to declare a non-atomic net (see 6.7).

### In 10.3.2  The continuous assignment statement, Add a new paragraph:

Nets can be driven by multiple continuous assignments or by a mixture of primitive outputs, module outputs, and continuous assignments. Variables can only be driven by one continuous assignment or by one primitive output or module output. It shall be an error for a variable driven by a continuous assignment or output to have an initializer in the declaration or any procedural assignment. See also 6.5.

A continuous assignment to an atomic net shall not drive part of the net; the entire nettype value shall be driven.Thus the left-hand side of a continuous assignment to a net of a user-defined nettype shall not contain any indexing or select operations into the data type of the nettype.

### At the end of 10.3.3 Continuous assignment delays, (add a new paragraph):

If the left-hand references a vector net, then up to three delays can be applied. The following rules determine which delay controls the assignment:

- If the right-hand side makes a transition from nonzero to zero, then the falling delay shall be used.

- If the right-hand side makes a transition to z, then the turn-off delay shall be used.

- For all other cases, the rising delay shall be used.

If the left hand references a user-defined net or an array of user-defined nets then only a single delay may be applied. The specific delay is used when any change occurs to the value of the net.

### In section 10.6.2 The force and release procedural statements, modify

The left-hand side of the assignment can be a reference to a singular variable, a

net, a constant bit-select of a vector net, a constant part-select of a vector net, or a concatenation of these. It shall not be a bit-select or a part-select of a variable. It shall not be a bit-select or part-select of an atomic net.

### In 23.3.3 Port connection rules, ADD at the end:

If the internal and external connections to a port are atomic nets, they shall be of matching nettypes and merged into a single simulated net. If only one of the two connections is an atomic net then they shall have matching data types, the port shall be of mode input or output and the connection shall be treated as a continuous assignment from source to sink.

### In 23.3.3.3, CHANGE CLAUSE TITLE TO:

23.3.3.3 Port connection rules for non atomic nets

### In 28.12 Strengths and values of combined signals, immediately before 28.12.1 Combined signals of unambiguous strength (ADD a new sentence):

In addition to a signal value, a net shall have either a single unambiguous strength level or an ambiguous strength consisting of more than one level. When signals combine, their strengths and values shall determine the strength and value of the resulting signal in accordance with the principles in 28.12.1 through 28.12.4. User-defined nets shall not have strength levels. Combining signal values for user-defined nets shall follow the rules in 23.12 (Resolution of user-defined nets). Any strength associated with any drivers of an atomic net shall be ignored.

## In Annex, A.2.1.3 Type declarations,

## ADD net_type_declaration in the production for data_declaration:

data_declaration9 ::=

[ const ] [ var ] [ lifetime ] data_type_or_implicit list_of_variable_decl_assignments ;

| type_declaration

| package_import_declaration10

| virtual_interface_declaration

| net_type_declaration

## ADD after type_declaration production:

type_declaration ::= …….

net_type_declaration ::=

nettype data_type  net_type_identifier

[with  [package_scope|class_scope] tf_identifier];

| nettype [package_scope|class_scope] net_type_identifier net_type_identifier;

## In Annex, A.9.3 Identifiers, ADD:

net_type_identifier ::= identifier

## In Annex, A.2.1.3 Type declarations, ADD:

net_type_identifier ::= identifier

## In Table B.1 – Reserved keywords, ADD:

nettype