### Details of Language Change

Changes shown in red font.  Deletions shown in strikethrough red font.  Comments shown in green font.

Twiki has not been kind in regards to font colors and strikethrough.  Word and PDF versions of these changes are attached as separate files to this LCS.

#### LRM 4.2.1 General (Subprogram declarations)

##### Page 19 near bottom

function_specification ::=

[ pure | impure ] function designator

subprogram_header

[ [ parameter ] ( formal_parameter_list ) ] return [identifier :] type_mark

##### Page 20 first paragraph

The specification of a procedure specifies its designator, its generics (if any), and its formal parameters (if any). The specification of a function specifies its designator, its generics (if any), its formal parameters (if any), the name that the function body may reference to retrieve attributes of the target (if any), the subtype of the returned value (the result subtype), and whether or not the function is pure. A function is impure if its specification contains the reserved word impure; otherwise, it is said to be pure. A procedure designator is always an identifier. A function designator is either an identifier or an operator symbol. A designator that is an operator symbol is used for the overloading of an operator (see 4.5.2). The sequence of characters represented by an operator symbol shall be an operator belonging to one of the classes of operators defined in 9.2. Extra spaces are not allowed in an operator symbol, and the case of letters is not significant.

##### Page 20 Add after the first paragraph

Add the following new material in entirety after the first paragraph (which ends with the sentence “Extra spaces are not allowed in an operator symbol, and the case of letters is not significant.”)

If the specification of a function includes an identifier prior to the result subtype, that identifier can be used within the function body as if it was a formal input variable of the result subtype of the function. The only allowable action within the function body with this identifier is to retrieve attributes. The attributes returned are the attributes of the target to which the output of the function is assigning. The named function with result identifier shall only be used in a context that meets one of the conditions of ‘Determinable index range conditions’ listed in section 9.3.3.3--Array aggregates. The attributes that may be referenced are:

- The elements BASE, LEFT, RIGHT, HIGH, LOW, ASCENDING, SUBTYPE defined in 16.2.2 (Predefined attributes of types and objects).

- All attributes defined in 16.2.3 Predefined attributes of arrays

It is an error to reference any other attributes.

#### LRM 9.3.3.3 Array aggregates

##### Page 134 middle of page

The index range of an array aggregate that has an others choice shall be determinable from the context by meeting one of the Determinable index range conditions. That is, an array aggregate with an others choice shall appear only in one of the following contexts:

Determinable index range conditions:

a) As an actual associated with a formal parameter, formal generic, or formal port (or member thereof), where either the formal (or the member) is declared to be of a fully constrained array subtype, or the formal designator is a slice name

#### LRM 12.2 Scope of Declarations

##### Page 186 middle of page

h) A declaration that occurs immediately within a protected type declaration

i) An architecture body

j) A return identifier after the formal parameter declaration in a subprogram declaration or specification

#### LRM Annex C Syntax Summary Changes

##### Page 489 near middle of the page

Changes are shown in red font.

interface_function_specification ::= [§ 6.5.4]
[ pure | impure ] function designator
[ [ parameter ] ( formal_parameter_list ) ] return [identifier :] type_mark